# Complete EKF Derivation for Indoor Navigation System (RSSI + IMU Fusion)

1. Problem Setup

4 ESP anchor nodes at known coordinates.

Robot has ESP32 + MPU-9250.

Anchors -> RSSI -> distance estimate -> trilateration -> (x,y).

IMU gives acceleration, gyro, magnetometer heading.

EKF fuses all.

2. State Vector

X = x, y, v, theta, omegaT

x,y = position

v = forward linear velocity

theta = heading yaw

omega = angular velocity

3. Continuous Model

dxdt= vcostheta

dydt= vsintheta

dvdt= a

dthetadt= omega

domegadt= 0

Angular velocity derivative assumed zero because over small timestep Δt angular rate changes little; unmodeled change enters process noise.

4. Euler Discretization

General:

Ifdsdt= fs,

then sk+1=sk+dsdtΔt

Applying:

xk+1=xk+vkcosthetakΔt

yk+1=yk+vksinthetakΔt

vk+1=vk+akΔt

thetak+1=thetak+omegakΔt

omegak+1=omegak

5. Process Model

Xk+1=fXk,uk+wk

6. Jacobian

Needed because system is nonlinear (sin/cos terms).

F =∂f∂X

Rows derived from partial derivatives:

1,0,costhetaΔt,-vsinthetaΔt,0

0,1,sinthetaΔt, vcosthetaΔt,0

0,0,1,0,0

0,0,0,1,Δt

0,0,0,0,1

7. Covariance

P is 5x5 uncertainty matrix.

Diagonal = variances of x,y,v,theta,omega.

8. Covariance Prediction

P-= F P FT+ Q

Q = process noise covariance.

9. Measurement Vector

z = xrssi, yrssi, thetamagT

Measurement function:

hX=x,y,thetaT

10. Residual

y = z - hX-

11. Measurement Jacobian

H =

1,0,0,0,0

0,1,0,0,0

0,0,0,1,0

12. Measurement Noise

R =

diagσx2, σy2, σtheta2

13. Innovation Covariance

S = H P-HT+ R

14. Kalman Gain

K = P-HTS-1

15. State Update

X = X-+ K y

16. Covariance Update

P = I-KHP-